# Smart Irrigation Pump Controller
Group Handoff Note

Current working version: relay + pump + standalone Wi-Fi web control

# 1. What is working right now

M5Stack Core2 controls the Grove Relay on GPIO 33.

The relay switches 12V power to the pump.

The pump has already been tested successfully with a short timed run.

The device connects to Wi-Fi and hosts a small web page for remote control.

After code upload, the Core2 can run without the laptop if powered by a phone charger or power bank.

# 2. Hardware wiring

Control side: laptop/USB power -> Core2, and Grove cable from Core2 to Grove Relay.

Pump power side: 12V adapter barrel plug -> female DC barrel jack adapter. Then DC jack + -> one relay terminal, other relay terminal -> pump positive terminal, and DC jack - -> pump negative terminal.

The relay board used here is the 2-terminal switched version, so there is no separate COM/NO/NC labeling needed in practice for our setup.

Relay logic confirmed: relay.value(1) = ON and relay.value(0) = OFF.

# 3. Files in flash

Keep these files inside /flash on the Core2. This is the clean version to share with the group.

config.py

RELAY_PIN = 33 RELAY_ON_VALUE = 1 RELAY_OFF_VALUE = 0 WIFI_SSID = "YOUR_WIFI_NAME" WIFI_PASSWORD = "YOUR_WIFI_PASSWORD" HTTP_PORT = 8080 DEFAULT_RUN_SECONDS = 3 MAX_RUN_SECONDS = 15

relay_control.py

from machine import Pin from config import RELAY_PIN, RELAY_ON_VALUE, RELAY_OFF_VALUE relay = Pin(RELAY_PIN, Pin.OUT) def init_relay(): relay.value(RELAY_OFF_VALUE) def relay_on(): relay.value(RELAY_ON_VALUE) print("Relay ON") def relay_off(): relay.value(RELAY_OFF_VALUE) print("Relay OFF") def relay_toggle(): relay.value(0 if relay.value() == RELAY_ON_VALUE else RELAY_ON_VALUE) print("Relay toggled. Raw value:", relay.value()) def relay_state(): return relay.value()

controller.py

import time from config import DEFAULT_RUN_SECONDS, MAX_RUN_SECONDS from relay_control import init_relay, relay_on, relay_off, relay_toggle, relay_state _mode = "OFF" _until_ms = None _last_action = "boot" def init_controller(): global _mode, _until_ms, _last_action init_relay() _mode = "OFF" _until_ms = None _last_action = "initialized" def pump_on(): global _mode, _until_ms, _last_action relay_on() _mode = "ON" _until_ms = None _last_action = "manual on" def pump_off(): global _mode, _until_ms, _last_action relay_off() _mode = "OFF" _until_ms = None _last_action = "manual off" def pump_toggle(): global _mode, _until_ms, _last_action relay_toggle() if relay_state() == 1: _mode = "ON" _until_ms = None _last_action = "toggle on" else: _mode = "OFF" _until_ms = None _last_action = "toggle off" def run_for(seconds=None): global _mode, _until_ms, _last_action if seconds is None: seconds = DEFAULT_RUN_SECONDS try: seconds = int(seconds) except: seconds = DEFAULT_RUN_SECONDS if seconds < 1: seconds = 1 if seconds > MAX_RUN_SECONDS: seconds = MAX_RUN_SECONDS relay_on() _mode = "TIMED" _until_ms = time.ticks_add(time.ticks_ms(), seconds * 1000) _last_action = "timed run {}s".format(seconds) def update_controller(): global _mode, _until_ms, _last_action if _until_ms is not None: if time.ticks_diff(_until_ms, time.ticks_ms()) <= 0: relay_off() _mode = "OFF" _until_ms = None _last_action = "timed run complete" def seconds_remaining(): if _until_ms is None: return None diff = time.ticks_diff(_until_ms, time.ticks_ms()) if diff <= 0: return 0 return diff // 1000 def status(): return { "mode": _mode, "relay_raw": relay_state(), "seconds_remaining": seconds_remaining(), "last_action": _last_action }

wifi_manager.py

import network import time from config import WIFI_SSID, WIFI_PASSWORD wlan = network.WLAN(network.STA_IF) def connect_wifi(timeout_s=20): if wlan.isconnected(): return wlan.ifconfig()[0] wlan.active(True) if not wlan.isconnected(): print("Connecting to Wi-Fi:", WIFI_SSID) wlan.connect(WIFI_SSID, WIFI_PASSWORD) start = time.time() while not wlan.isconnected(): if time.time() - start > timeout_s: raise Exception("Wi-Fi connection timed out") time.sleep(1) ip = wlan.ifconfig()[0] print("Wi-Fi connected") print("IP:", ip) return ip def get_ip(): if wlan.isconnected(): return wlan.ifconfig()[0] return None

web_server.py

import socket from controller import pump_on, pump_off, pump_toggle, run_for, status, update_controller from config import HTTP_PORT def parse_path(request_text): try: first_line = request_text.split("\r\n")[0] parts = first_line.split() if len(parts) >= 2: return parts[1] except: pass return "/" def parse_run_seconds(path): if "seconds=" not in path: return None try: value = path.split("seconds=")[1].split("&")[0] return int(value) except: return None def html_page(): s = status() remaining = s["seconds_remaining"] if remaining is None: remaining_text = "N/A" else: remaining_text = str(remaining) return """<!DOCTYPE html> <html> <head> <title>Smart Irrigation Controller</title> <meta name="viewport" content="width=device-width, initial-scale=1"> <meta http-equiv="refresh" content="2"> <style> body { font-family: Arial, sans-serif; text-align: center; padding: 20px; background: #f4f4f4; } .card { max-width: 420px; margin: auto; background: white; padding: 20px; border-radius: 16px; box-shadow: 0 4px 16px rgba(0,0,0,0.12); } .btn { display: inline-block; margin: 8px; padding: 14px 22px; text-decoration: none; color: white; border-radius: 12px; font-size: 18px; } .on { background: #2e7d32; } .off { background: #c62828; } .toggle { background: #1565c0; } .run { background: #6a1b9a; } .status { text-align: left; margin-top: 20px; font-size: 17px; line-height: 1.8; } </style> </head> <body> <div class="card"> <h2>Smart Irrigation Controller</h2> <a class="btn on" href="/on">ON</a> <a class="btn off" href="/off">OFF</a> <a class="btn toggle" href="/toggle">TOGGLE</a> <br><br> <a class="btn run" href="/run?seconds=2">RUN 2s</a> <a class="btn run" href="/run?seconds=5">RUN 5s</a> <a class="btn run" href="/run?seconds=10">RUN 10s</a> <div class="status"> <b>Mode:</b> {mode}<br> <b>Relay raw:</b> {relay_raw}<br> <b>Seconds remaining:</b> {remaining}<br> <b>Last action:</b> {last_action}<br> </div> </div> </body> </html> """.format( mode=s["mode"], relay_raw=s["relay_raw"], remaining=remaining_text, last_action=s["last_action"] ) def http_response(body): return "HTTP/1.1 200 OK\r\nContent-Type: text/html\r\nConnection: close\r\n\r\n" + body def handle_path(path): if path == "/on": pump_on() elif path == "/off": pump_off() elif path == "/toggle": pump_toggle() elif path.startswith("/run"): seconds = parse_run_seconds(path) run_for(seconds) def start_server(): addr = socket.getaddrinfo("0.0.0.0", HTTP_PORT)[0][-1] s = socket.socket() try: s.setsockopt(socket.SOL_SOCKET, socket.SO_REUSEADDR, 1) except: pass s.bind(addr) s.listen(2) s.settimeout(0.2) print("Web server listening on port", HTTP_PORT) return s def serve_forever(server_socket): while True: update_controller() try: client, addr = server_socket.accept() except OSError: continue try: request = client.recv(1024) request_text = request.decode("utf-8", "ignore") path = parse_path(request_text) print("Request path:", path) handle_path(path) body = html_page() response = http_response(body) client.send(response.encode()) except Exception as e: print("Client error:", e) finally: try: client.close() except: pass

main.py

from controller import init_controller from wifi_manager import connect_wifi from web_server import start_server, serve_forever from config import HTTP_PORT print("Booting smart irrigation controller...") init_controller() ip = connect_wifi() print("Open this in your browser:") print("http://{}:{}/".format(ip, HTTP_PORT)) server = None try: server = start_server() serve_forever(server) finally: if server is not None: try: server.close() except: pass

# 4. How to run

Upload all six files into /flash.

Edit config.py and put the correct Wi-Fi SSID and password.

Run main.py once while connected to the laptop.

Read the IP address printed in the console.

Open http://DEVICE_IP:8080/ from a phone or laptop on the same Wi-Fi.

After that, the Core2 can be powered by a normal USB charger or power bank and still work without the laptop.

# 5. Small notes for the group

If you see EADDRINUSE, soft reset the board or reboot it. Port 8080 worked better than 80 for us.

Keep relay default OFF on boot for safety.

Do only short pump tests for now.

Current project status is relay + pump + standalone web control. Smart scheduling, BLE flow sensing, MQTT, and ML are the next phases.

# 6. Next phase plan

Next we should add: (1) schedule with internet time, (2) save schedule to JSON, (3) BLE flow-sensor node, (4) rule-based dry-run shutdown, and only then (5) tiny ML fault detection.